DVT comparator HT

Design based Volga device from CISSOID

| Version | Date | Created by / at |
| --- | --- | --- |
| 1.00 | 2016-08-24 | S.Kulik 2016-08-24 |

# CISSOID Volga

Main features

- Temperature range: -55°C to 225°C (and still on-going)

- Validated at 225°C for 1000 hours

- Rail to Rail I/O

- Internal hysteresis: 6mV

- Push-Pull CMOS output stage +/- 16mA max

- Maximum operating frequency: 25MHz

- Supply rating: 4.5-5.5Vcc

DESCRIPTION

The CHT-VOLGA is a single high-speed comparator with a normal operating tem-perature range -55°C to +225°C.

The device operates from a single +5V±10% power supply, with rail-to-rail in-put / output. CHT-VOLGA features a shut-down mode, controllable through an Enable digital input pin that places the device in a low power consumption mode when the comparator function is not needed. The comparator features an internal hysteresis (6mV Typ.) for improved noise immunity.

The output stage implements a push-pull CMOS stage, that can sink /or source up to 16mA.

The CHT-VOLGA is available in a tiny TDFP16 surface mount package. The TDFP package is an SMD solution with leads, available by default in 16-pin.

Functional diagram

Schematic

PCBs

CHT-7400 inverted Schmitt-Trigger device connected on output from comparator can be used to better signal form condition.

On workbench all 5 Volga devices tested with 5 and 3.3 Volt supply and have same performance up to 3.5MHz frequency.

Color legend:

Yellow (1): direct comparator output

Cyan (2): CHT-7400 buffered output

200kHz @ 5V

200kHz @ 3.3V

400kHz @ 5V

400kHz @ 3.3V

600kHz @ 5V

600kHz @ 3.3V

800kHz @ 5V

800kHz @ 3.3V

1MHz @ 5V

1MHz @ 3.3V

1.5MHz @ 5V

1.5MHz @ 3.3V

2MHz @ 5V

2MHz @ 3.3V

2.5MHz @ 5V

2.5MHz @ 3.3V

3MHz @ 5V

3MHz @ 3.3V

3.5MHz @ 5V

3.5MHz @ 3.3V

Summary

Atlas device works very well and in first tests works stable as well